Free Vibrations of Simply Supported Beam with Distributed Mass

.

AI generated calculations (as continuous mass system)

Dynamic analysis based on Euler-Bernoulli beam theory

1. Input Data

Beam geometry

Span length - L=6 m

Cross-section (rectangular)

Width - b=0.3 m

Height - h=0.5 m

Cross-section area - A=b·h=0.3 m·0.5 m=0.15 m2

Second moment of area - I=b·h312=0.3 m·0.5 m312=0.00313 m4

Material properties

Modulus of elasticity - E=30 GPa

Mass density - ρ=2500 kgm3

Derived quantities

Bending stiffness - EI=E·I=30 GPa·0.00313 m4=0.0938 Mt·m3∕s2

Distributed mass per unit length - m=ρ·A=2500 kg∕m3·0.15 m2=375 kg∕m

Total mass of the beam - Mtot=m·L=375 kg∕m·6 m=2250 kg

2. Analytical Solution

Theory

For a simply supported beam with uniformly distributed mass, the equation of free vibrations is:

EI·∂⁴w/∂x⁴ + m·∂²w/∂t² = 0

The solution is obtained by separation of variables:

w(x, t) = φ(x)·q(t)

Mode shapes: φn(x) = sin(nπx/L)

Natural circular frequencies:

ωn = (nπ/L)² · √(EI/m)

Number of modes to compute

n=5

3. Natural Frequencies and Periods

Circular frequencies ωn

ωk=k·πL2·EIm

Natural frequencies fn

fk=ωk2·π

Vibration periods Tn

Tk=1fk

Results for the first  n=5  modes:

| Mode | ωn [rad/s] | fn [Hz] | Tn [s] |
| --- | --- | --- | --- |
| 1 | 137.08 | 21.82 | 0.05 |
| 2 | 548.31 | 87.27 | 0.01 |
| 3 | 1233.7 | 196.35 | 0.01 |
| 4 | 2193.25 | 349.07 | 0 |
| 5 | 3426.95 | 545.42 | 0 |

Fundamental frequency (1st mode)

Circular frequency - ω1=ω1=137.08 Hz

Natural frequency - f1=f1=21.82 Hz

Vibration period - T1=T1=0.0458 s

4. Mode Shapes

The mode shapes are sinusoidal functions:

φn(x) = sin(nπx/L)

φk;x=sink·π·xL

PlotHeight=120

5. Modal Properties

Modal mass

For sinusoidal mode shapes of a simply supported beam:

Mn = ∫₀ᴸ m·φn²(x) dx = m·L/2

Mn=m·L2=375 kg∕m·6 m2=1125 kg

Modal stiffness

Kn = ωn²·Mn

| Mode | Mn [kg] | Kn [kN/m] |
| --- | --- | --- |
| 1 | 1125 | 21139.13 |
| 2 | 1125 | 338226.01 |
| 3 | 1125 | 1712269.18 |
| 4 | 1125 | 5411616.17 |
| 5 | 1125 | 13211953.54 |

6. Free Vibration Response

Initial conditions

Initial midspan displacement - w0=5 mm

Initial velocity - v0=0 ms

Response at midspan

For initial displacement w₀ at midspan with zero initial velocity,

the modal initial conditions are:

qn(0) = (2/L)·∫₀ᴸ w₀·δ(x-L/2)·sin(nπx/L) dx

For concentrated initial displacement at midspan:

qn(0) = w₀·sin(nπ/2)·(2/(m·L))

Only odd modes are excited (modes 1, 3, 5,...).

Time history of midspan displacement (first mode approximation)

Amplitude of first mode - A1=w0=5 mm

Response - wt=A1·sinπ·L2L·cosω1·t

tmax=3·T1=0.138 s

7. Verification

Comparison with approximate formulas

Static deflection under self-weight:

δst = 5·m·g·L⁴/(384·EI)

g=9.81 ms2

δst=5·m·g·L4384·EI=5·375 kg∕m·9.81 m∕s2·6 m4384·0.0938 Mt·m3∕s2=0.662 mm

Approximate fundamental frequency (Rayleigh method):

f1R=12·π·gδst=12·3.14·9.81 m∕s20.662 mm=19.37 s-1

Exact fundamental frequency: f₁ = π/(2L²)·√(EI/m)

Ratio (exact/Rayleigh) - r=f1f1R=21.82 Hz19.37 s-1=1.13

The Rayleigh method gives a slightly different result because it uses a static deflection shape (parabolic) instead of the exact sinusoidal mode shape.

8. Frequency Ratios

Relationship between higher modes and fundamental frequency

For a simply supported beam: fn/f1 = n²

| Mode n | fn/f1 | n² |
| --- | --- | --- |
| 1 | 1 | 1 |
| 2 | 4 | 4 |
| 3 | 9 | 9 |
| 4 | 16 | 16 |
| 5 | 25 | 25 |

This confirms the characteristic property of simply supported beams:

natural frequencies increase proportionally to the square of the mode number.

Manual verification (as equivalent discrete mass SDOF or  MDOF systems)

.

Structure type - simply supported beam

Beam length - L=6 m

Cross section - rectangular with dimensions:

Width - b=300 mm

Height - h=500 mm

Area - A=b·h=300 mm·500 mm=1500 cm2

Second moment of area - I=b·h312=300 mm·500 mm312=3125000000 mm4

Shear area - AQ=56·A=56·1500 cm2=1250 cm2

Modulus of elasticity - E=30 GPa

Poisson′s ratio - ν=0.2

Shear modulus - G=E2·1+ν=30 GPa2·1+0.2=12.5 GPa

Mass density - ρ=2500 kgm3

Distributed mass per unit length - m=ρ·A=2500 kg∕m3·1500 cm2=375 kg∕m

Simple solution as equivalent SDOF system

The structure is reduced to a SDOF system for simplicity

Dynamically equivalent mass - Mtot=2·Lπ·m=2·6 m3.14·375 kg∕m=1432.39 kg

Structural stiffness for a vertical force applied at the middle point of the span

K=48·E·IL3=48·30 GPa·3125000000 mm46 m3=20833.3 kN∕m

Natural circular frequency - ω1=KMtot=20833.3 kN∕m1432.39 kg=120.6 s-1

Vibration frequency - f1=ω12·π=120.6 s-12·3.14=19.19 s-1

Vibration period - T1=2·πω1=2·3.14120.6 s-1=0.0521 s

Solution as an MDOF system

Number of intermediate joints - nJ=11

Length of one segment - Δx=LnJ+1=6 m11+1=0.5 m

Coordinate of joint j - xJj=Δx·j

Shear forces due to unit vertical load Fj = 1 at joint j

V1x;j= if x<xJj:1-xJjL else:-xJjL

Bending moments due to unit vertical load Fj = 1 at joint j

M1,maxj=xJjL-1·xJj

M1x;j=M1,maxj· if x<xJj:xxJj else:L-xL-xJj

Flexibility matrix

Di;j=0 mLM1x;i·M1x;j dx·1E·I + $Area{V_1(x; i)*V_1(x; j) @ x = 0m : L}*(1/(G*A_Q)) (skipped for compatibility)

D=0.004480.007960.01030.01170.01220.01190.01090.009410.007440.005150.002630.007960.01480.01970.02250.02360.02310.02130.01840.01460.01010.005150.01030.01970.0270.03160.03340.0330.03060.02640.0210.01460.007440.01170.02250.03160.03790.0410.04090.03810.03320.02640.01840.009410.01220.02360.03340.0410.04540.04610.04350.03810.03060.02130.01090.01190.02310.0330.04090.04610.0480.04610.04090.0330.02310.01190.01090.02130.03060.03810.04350.04610.04540.0410.03340.02360.01220.009410.01840.02640.03320.03810.04090.0410.03790.03160.02250.01170.007440.01460.0210.02640.03060.0330.03340.03160.0270.01970.01030.005150.01010.01460.01840.02130.02310.02360.02250.01970.01480.007960.002630.005150.007440.009410.01090.01190.01220.01170.01030.007960.00448 mm/kN

Mass matrix

dM.j=m·Δxt=375 kg∕m·0.5 mt=0.188

M=0.188000000000000.188000000000000.188000000000000.188000000000000.188000000000000.188000000000000.188000000000000.188000000000000.188000000000000.188000000000000.188

Total mass of the structure - sumdM=2.06 t

Eigenvalues

Msq=M=0.433000000000000.433000000000000.433000000000000.433000000000000.433000000000000.433000000000000.433000000000000.433000000000000.433000000000000.433000000000000.433

C=copyMsq·D·Msq;symmetricnJ;1;1=copyMsq·D·Msq;symmetric11;1;1=0.000840.001490.001940.002190.002280.002230.002050.001760.00140.0009650.0004930.001490.002780.003690.004220.004420.004330.003990.003440.002730.001890.0009650.001940.003690.005060.005920.006270.006190.005730.004960.003940.002730.00140.002190.004220.005920.007110.007680.007670.007150.006220.004960.003440.001760.002280.004420.006270.007680.008510.008650.008160.007150.005730.003990.002050.002230.004330.006190.007670.008650.0090.008650.007670.006190.004330.002230.002050.003990.005730.007150.008160.008650.008510.007680.006270.004420.002280.001760.003440.004960.006220.007150.007670.007680.007110.005920.004220.002190.00140.002730.003940.004960.005730.006190.006270.005920.005060.003690.001940.0009650.001890.002730.003440.003990.004330.004420.004220.003690.002780.001490.0004930.0009650.00140.001760.002050.002230.002280.002190.001940.001490.00084

λ=eigenvalsC·10-3;-7=5.32×10-5 3.33×10-6 6.57×10-7 2.08×10-7 8.57×10-8 4.17×10-8 2.29×10-8

Natural circular frequences - ω=1λ=137.08 548.28 1233.32 2190.89 3416.68 4898.98 6609.4 s⁻¹

Natural vibration frequences - f=ω2·π·Hz=ω2·3.14·Hz=21.82Hz 87.26Hz 196.29Hz 348.69Hz 543.78Hz 779.7Hz 1051.92Hz

Natural vibration periods - T=1f=0.0458s 0.0115s 0.00509s 0.00287s 0.00184s 0.00128s 0.000951s

Eigenvectors

V=transpeigenvecsC·10-3;-7=0.1060.2040.2890.3540.3940.4080.3940.2040.3540.4080.3540.204-6×10-14-0.2040.2890.4080.2890-0.289-0.408-0.2890.3540.3540-0.354-0.3549.54×10-140.3540.3940.204-0.289-0.3540.1060.4080.1060.4080-0.40800.408-1.04×10-14-0.4080.394-0.204-0.2890.3540.106-0.4080.1060.354-0.35400.354-0.3548.73×10-150.3540.289-0.4080.289-1.48×10-14-0.2890.408-0.2890.204-0.3540.408-0.3540.204-5.96×10-14-0.2040.106-0.2040.289-0.3540.394-0.4080.394

Φ=inverseMsq·V=0.2440.4710.6670.8160.9110.9430.9110.4710.8160.9430.8160.471-1.39×10-13-0.4710.6670.9430.6670-0.667-0.943-0.6670.8160.8160-0.816-0.8162.2×10-130.8160.9110.471-0.667-0.8160.2440.9430.2440.9430-0.94300.943-2.39×10-14-0.9430.911-0.471-0.6670.8160.244-0.9430.2440.816-0.81600.816-0.8162.02×10-140.8160.667-0.9430.667-3.41×10-14-0.6670.943-0.6670.471-0.8160.943-0.8160.471-1.38×10-13-0.4710.244-0.4710.667-0.8160.911-0.9430.911

X=stackmatrix1;3;Φ;matrix1;3=00000000.2440.4710.6670.8160.9110.9430.9110.4710.8160.9430.8160.471-1.39×10-13-0.4710.6670.9430.6670-0.667-0.943-0.6670.8160.8160-0.816-0.8162.2×10-130.8160.9110.471-0.667-0.8160.2440.9430.2440.9430-0.94300.943-2.39×10-14-0.9430.911-0.471-0.6670.8160.244-0.9430.2440.816-0.81600.816-0.8162.02×10-140.8160.667-0.9430.667-3.41×10-14-0.6670.943-0.6670.471-0.8160.943-0.8160.471-1.38×10-13-0.4710.244-0.4710.667-0.8160.911-0.9430.9110000000

Comparison of the results

| AI gen, Hz | Manual, Hz | Difference |
| --- | --- | --- |
| 21.82 | 21.82 | 0.00% |
| 87.27 | 87.26 | 0.01% |
| 196.35 | 196.29 | 0.03% |
| 349.07 | 348.69 | 0.11% |
| 545.42 | 543.78 | 0.30% |