Finite Element Analysis of Deep Beam with Calcpad

| ## Input data Length - l=4 m, Height - h=2 m Thickness - t=0.1 m Loads Distributed load - q=100 kN/m Load length - b=0.8 m Load position - a=l-b2=4-0.82=1.6 m Load function - qx=q·x≥a and x≤a+b |  |
| --- | --- |

Supports

Support length - d=0.4 m

Support elastic stiffness - ks=50000 MN/m²

Support function - sx=ks·t·x≤d or x≥l-d

Material properties

Modulus of elasticity - E=20000 MPa

Poisson`s ratio - ν=0.2

## Finite element mesh

We will use rectangular finite element with n=8 DOFs

Number of elements along l and h - nl=20 ,  nh=10

Total number of elements - ne=nl·nh=20·10=200

Total number of joints - nj=nl+1·nh+1=20+1·10+1=231

Element dimensions - l1=lnl=420=0.2,   h1=hnh=210=0.2

Joint coordinates

xj=0 0 0 0 0 0 0 0 0 0 0 0.2 0.2 0.2 0.2 0.2 0.2 0.2 0.2 0.2 ... 4 m

yj=0 0.2 0.4 0.6 0.8 1 1.2 1.4 1.6 1.8 2 0 0.2 0.4 0.6 0.8 1 1.2 1.4 1.6 ... 2 m

Joint numbers for elements

transpej=1234567891012131415161718192021⋯21923456789101113141516171819202122⋯2201213141516171819202123242526272829303132⋯2301314151617181920212224252627282930313233⋯231

Coordinates of element centers

jee=rowej;e,   xce=sumextractxj;jee4,   ycesumextractyj;jee4

xc=0.1 0.1 0.1 0.1 0.1 0.1 0.1 0.1 0.1 0.1 0.3 0.3 0.3 0.3 0.3 0.3 0.3 0.3 0.3 0.3 … 3.9 m

yc=0.1 0.3 0.5 0.7 0.9 1.1 1.3 1.5 1.7 1.9 0.1 0.3 0.5 0.7 0.9 1.1 1.3 1.5 1.7 1.9 … 1.9 m

Elements along the bottom (supported) edge - eS=findltyc;h1;1=findltyc;0.2;1=1 11 21 31 41 51 61 71 81 91 101 111 121 131 141 151 161 171 181 191

Elements along the top (loaded) edge - eL=findgtyc;h-h1;1=findgtyc;2-0.2;1=10 20 30 40 50 60 70 80 90 100 110 120 130 140 150 160 170 180 190 200

## Finite element formation

Shape functions

They are defined in relative coordinates to the center of the element:

⚓

ξ=2·x-xcl1∈ (-1; 1),   η=2·y-ych1 ∈ (-1; 1)

Base functions -  Φ1ξ=1-ξ2, Φ2ξ=1+ξ2

First derivatives - Φ'1ξ=-12, Φ'2ξ=12

Shape functions for joints at elements` corners

N1ξ;η=Φ1ξ·Φ1η ,  N2ξ;η=Φ1ξ·Φ2η

N3ξ;η=Φ2ξ·Φ1η ,  N4ξ;η=Φ2ξ·Φ2η

Constitutive matrix (stress-strain relationship)

D=E·t1-ν2·1;ν;0 | ν;1;0 | 0;0;1-ν2=20000·0.11-0.22·1;0.2;0 | 0.2;1;0 | 0;0;1-0.22 =

2083.33416.670416.672083.33000833.33

Strain-displacement matrix

B1j;η=1l1·takej;-Φ1η;0;-Φ2η;0;Φ1η;0;Φ2η;0

B2j;ξ=1h1·takej;0;-Φ1ξ;0;Φ1ξ;0;-Φ2ξ;0;Φ2ξ

B3j;ξ;η=takej;-Φ1ξh1;-Φ1ηl1;Φ1ξh1;-Φ2ηl1;-Φ2ξh1;Φ1ηl1;Φ2ξh1;Φ2ηl1

Bj;ξ;η=B1j;η;B2j;ξ;B3j;ξ;η

The elements of the stiffness matrix will be calculated by using direct integration

Ke,ij=l1·h14·-11-11Biξ;ηT·D·Bjξ;η dξdη

Element stiffness matrix (above the main diagonal only)

BTDBei;j;ξ;η=transpBi;ξ;η·D·Bj;ξ;η

Kei;j=l1·h14·-11-11BTDBei;j;ξ;η dξdη

$Repeat$RepeatKe.i,j=Kei;j;j=i...n;i=1...n

Ke=972.22312.569.44104.17-555.56-104.17-486.11-312.50972.22-104.17-555.56104.1769.44-312.5-486.1100972.22-312.5-486.11312.5-555.56104.17000972.22312.5-486.11-104.1769.440000972.22-312.569.44-104.1700000972.22104.17-555.56000000972.22312.50000000972.22 MN/m

Boundary conditions

Supports

Number of elements along the supported edge - nS=leneS=20

Element′s joint springs stiffness factors

Ks,je=l12·-11Njξ;-1·sxc.e+ξ·l12dξ,   j=1 , 3

Results for element 1

Ks,1e=l12·-11N1ξ;-1·sxc.e+ξ·l12dξ,   Ks,11=500 MN/m

Ks,3e=l12·-11N3ξ;-1·sxc.e+ξ·l12dξ,   Ks,31=500MN/m

Loads

Number of elements along the loaded edge - nL=leneL=20

Element load vector

Fje=l12·-11Njξ;1·qxc.e+ξ·l12 dξ,   j=2 , 4

Results for element 100

F2e=l12·-11N2ξ;1·qxc.e+ξ·l12 dξ,   F2e=F2100=10 kN

F4e=l12·-11N4ξ;1·qxc.e+ξ·l12 dξ,   F4e=F4100=10 kN

## Solution

Global stiffness matrix - K=

972.22312.569.44104.170000000000000000⋯0312.51472.22-104.17-555.560000000000000000⋯069.44-104.171944.44069.44104.1700000000000000⋯0104.17-555.5601944.44-104.17-555.5600000000000000⋯00069.44-104.171944.44069.44104.17000000000000⋯000104.17-555.5601944.44-104.17-555.56000000000000⋯0000069.44-104.171944.44069.44104.170000000000⋯00000104.17-555.5601944.44-104.17-555.560000000000⋯000000069.44-104.171944.44069.44104.1700000000⋯0000000104.17-555.5601944.44-104.17-555.5600000000⋯00000000069.44-104.171944.44069.44104.17000000⋯000000000104.17-555.5601944.44-104.17-555.56000000⋯0000000000069.44-104.171944.44069.44104.170000⋯00000000000104.17-555.5601944.44-104.17-555.560000⋯000000000000069.44-104.171944.44069.44104.1700⋯0000000000000104.17-555.5601944.44-104.17-555.5600⋯00000000000000069.44-104.171944.44069.44104.17⋯000000000000000104.17-555.5601944.44-104.17-555.56⋯0000000000000000069.44-104.171944.440⋯00000000000000000104.17-555.5601944.44⋯0⋮⋮⋮⋮⋮⋮⋮⋮⋮⋮⋮⋮⋮⋮⋮⋮⋮⋮⋮⋮⋱⋮00000000000000000000⋯972.22

Global load vector

F=0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 ... 0

sumF=80 kN

Solution of the system of equations - Z=clsolveK;F=

[0.0543  0.00979  0.0429  0.0144  0.0296  0.0215  0.0185  0.0288  0.00958  0.0348 0.00188…

… 0.0391    -0.00543  0.0419   -0.013  0.0433   -0.0212  0.0438    -0.0302 0.0438  … 0.0438] mm

## Results

Horizontal joint displacements, ·10-3mm - transpu =

54.354.3955.2252.9747.4241.1334.1726.4718.099.190-9.19-18.09-26.47-34.17-41.13-47.42-52.97-55.22-54.39-54.342.9140.9137.6735.333.2129.5724.8819.4113.316.770-6.77-13.31-19.41-24.88-29.57-33.21-35.3-37.67-40.91-42.9129.5627.7725.6823.7722.1320.1117.2513.659.464.840-4.84-9.46-13.65-17.25-20.11-22.13-23.77-25.68-27.77-29.5618.4917.215.9514.8213.8312.7111.169.046.373.30-3.3-6.37-9.04-11.16-12.71-13.83-14.82-15.95-17.2-18.499.588.657.947.437.16.796.255.313.892.070-2.07-3.89-5.31-6.25-6.79-7.1-7.43-7.94-8.65-9.581.881.270.9250.9071.191.632.032.141.811.050-1.05-1.81-2.14-2.03-1.63-1.19-0.907-0.925-1.27-1.88-5.43-5.76-5.82-5.44-4.6-3.41-2.08-0.893-0.1280.1160-0.1160.1280.8932.083.414.65.445.825.765.43-12.99-13.1-12.92-12.22-10.93-9.05-6.74-4.34-2.31-0.91500.9152.314.346.749.0510.9312.2212.9213.112.99-21.19-21.16-20.82-19.94-18.38-16.04-12.84-9.07-5.42-2.4202.425.429.0712.8416.0418.3819.9420.8221.1621.19-30.16-30.09-29.75-28.91-27.4-25.04-21.59-16.6-10.82-5.4805.4810.8216.621.5925.0427.428.9129.7530.0930.16-39.62-39.67-39.67-39.29-38.21-36.28-33.46-29.67-23.25-12.73012.7323.2529.6733.4636.2838.2139.2939.6739.6739.62

Vertical joint displacements, ·10-3mm - transpv =

9.7917.9334.3562.3679.3793.17104.42113.38119.96124.01125.37124.01119.96113.38104.4293.1779.3762.3634.3517.939.7914.4526.7742.8362.8680.3894.24105.67114.8121.52125.65127.04125.65121.52114.8105.6794.2480.3862.8642.8326.7714.4521.4734.1948.8565.2481.1595.02106.6115.93122.83127.09128.53127.09122.83115.93106.695.0281.1565.2448.8534.1921.4728.7739.9853.2967.8182.3495.77107.44117.01124.18128.63130.14128.63124.18117.01107.4495.7782.3467.8153.2939.9828.7734.7844.5956.770.0383.6396.63108.34118.22125.77130.53132.16130.53125.77118.22108.3496.6383.6370.0356.744.5934.7839.1348.0959.3171.8184.7797.5109.34119.67127.79133.01134.82133.01127.79119.67109.3497.584.7771.8159.3148.0939.1341.8850.5461.273.185.6198.2110.33121.34130.33136.31138.41136.31130.33121.34110.3398.285.6173.161.250.5441.8843.352.0962.4373.8986.0698.58111.11123.07133.41140.59143.18140.59133.41123.07111.1198.5886.0673.8962.4352.0943.343.7952.9163.0974.2386.1198.52111.37124.52136.88146.02149.4146.02136.88124.52111.3798.5286.1174.2363.0952.9143.7943.853.2363.3274.2485.8898.1110.9125.06140.48152.68157.05152.68140.48125.06110.998.185.8874.2463.3253.2343.843.7553.363.3374.1185.5697.58110.24124.05143.81160.41164.51160.41143.81124.05110.2497.5885.5674.1163.3353.343.75

Calculation of internal forces

Displacements for joint -  Zjj=sliceZ;k1·j-1+1;k1·j

Displacements for element -  Zee=Zjej.e,1;Zjej.e,2;Zjej.e,3;Zjej.e,4

Membrane forces in element - Nee;x;y=-D·B2·xl1;2·yh1·Zee

Results for element 101 and joint 111:

Ze=Zee=Ze101=0 0.125 0 0.127 -0.00919 0.124 -0.00677 0.126 mm

Ne=Nee;x;y=Ne101;-0.1;-0.1=92.26 1.73 5.68 kN/m

Average membrane forces at joints, kN/m - Nj =

-10.658.693.69-0.36-1.12-1.08-0.904-0.815-0.841-0.7520.592-23.1910.356.472.410.122-1.24-2.17-2.82-3.09⋯0.592-48.71- 56.68-70.86-66.62-52.02-35.69-21.04-9.71-2.660.02120.584-93.05-79.26-64.78-51.47-40.48-30.04-20.44-12.39-6.35⋯0.58413.550.204-2.13-5.1-6.23-6.04-5.13-3.79-2.23-0.9-0.3765-3.67-7.63-11.24-12.49-12.01-10.32-7.8-4.84⋯0.376

Membrane forces for the structure

Normal membrane forces - Nx, kN/m - transpNx =

-10.65-23.19-10.2639.5659.5666.8273.7380.7686.7890.8392.2690.8386.7880.7673.7366.8259.5639.56-10.26-23.19-10.658.6910.3514.1420.2227.9941.4750.6557.6462.8366.0967.2166.0962.8357.6450.6541.4727.9920.2214.1410.358.693.696.479.9613.31723.8131.8138.3243.1446.1447.1646.1443.1438.3231.8123.811713.39.966.473.69-0.362.414.26.068.3912.2217.3422.5626.8229.6130.5829.6126.8222.5617.3412.228.396.064.22.41-0.36-1.120.1220.0800.1870.8442.655.729.513.1215.7116.6515.7113.129.55.722.650.8440.1870.0800.122-1.12-1.08-1.24-2.79-4.56-5.84-6-4.7-2.140.933.434.393.430.93-2.14-4.7-6-5.84-4.56-2.79-1.24-1.08-0.904-2.17-4.96-8.51-11.89-14.27-14.96-13.7-11.11-8.56-7.49-8.56-11.11-13.7-14.96-14.27-11.89-8.51-4.96-2.17-0.904-0.815-2.82-6.55-11.57-17.05-22.13-25.57-26.37-24.69-22.16-20.98-22.16-24.69-26.37-25.57-22.13-17.05-11.57-6.55-2.82-0.815-0.841-3.09-7.25-13.08-20.15-28.34-36.09-40.74-42.01-40.81-39.61-40.81-42.01-40.74-36.09-28.34-20.15-13.08-7.25-3.09-0.841-0.752-2.58-6.36-12.07-19.61-29.3-42.76-55.56-65.15-71.38-72.8-71.38-65.15-55.56-42.76-29.3-19.61-12.07-6.36-2.58-0.7520.5920.122-2-7.34-15.02-23.66-33.05-51.1-95.19-137.19-148.15-137.19-95.19-51.1-33.05-23.66-15.02-7.34-20.1220.592

Plot for Nx, kN/m at x = l/2

Bottom value - Nxl2;0=Nx42;0=92.26 kN/m

Top value - Nxl2;h=Nx42;2=-148.15 kN/m

Normal membrane forces - Ny, kN/m - transpNy =

-48.71-93.05-86.82.921.762.72.221.851.761.731.761.8522.22.71.762.92-86.8-93.05-48.71-56.68-79.26-69.65-10.32-3.3-0.933-0.794-1.23-1.78-2.2-2.36-2.2-1.78-1.23-0.794-0.933-3.3-10.32-69.65-79.26-56.68-70.86-64.78-50.3-22.09-6.41-2.88-2.47-3.41-4.68-5.69-6.07-5.69-4.68-3.41-2.47-2.88-6.41-22.09-50.3-64.78-70.86-66.62-51.47-38.41-22.78-10.75-5.63-5.24-6.94-9.32-11.26-12.01-11.26-9.32-6.94-5.24-5.63-10.75-22.78-38.41-51.47-66.62-52.02-40.48-30.1-19.93-11.95-8.11-8.38-11.4-15.44-18.78-20.08-18.78-15.44-11.4-8.38-8.11-11.95-19.93-30.1-40.48-52.02-35.69-30.04-23.07-16.21-11.05-9.07-10.89-15.99-22.62-28.21-30.39-28.21-22.62-15.99-10.89-9.07-11.05-16.21-23.07-30.04-35.69-21.04-20.44-16.6-12.13-8.84-8.3-11.8-19.72-30.32-39.58-43.26-39.58-30.32-19.72-11.8-8.3-8.84-12.13-16.6-20.44-21.04-9.71-12.39-10.77-8-5.93-5.98-10.27-21.2-37.7-52.98-59.15-52.98-37.7-21.2-10.27-5.98-5.93-8-10.77-12.39-9.71-2.66-6.35-5.89-4.35-3.11-3.23-6.19-18.09-43.78-68.64-77.28-68.64-43.78-18.09-6.19-3.23-3.11-4.35-5.89-6.35-2.660.0212-2.5-2.47-1.77-1.17-1.16-2.94-8.75-47.68-86.25-90.13-86.25-47.68-8.75-2.94-1.16-1.17-1.77-2.47-2.50.02120.584-0.693-0.515-0.1410.1390.461-0.0283-0.167-52.35-104.7-104.3-104.7-52.35-0.167-0.02830.4610.139-0.141-0.515-0.6930.584

Top value - Nyl2;h=Ny42;2=-104.3 kN/m

Shear membrane forces - Nxy, kN/m - transpNxy =

13.555-19.47-20.12-4.98-4.01-3.42-2.94-2.18-1.1701.172.182.943.424.014.9820.1219.47-5-13.550.204-3.67-13.65-17.4-12.68-8.91-7.6-6.3-4.6-2.4402.444.66.37.68.9112.6817.413.653.67-0.204-2.13-7.63-19.42-24.62-21.66-17.92-14.99-12.21-8.8-4.6404.648.812.2114.9917.9221.6624.6219.427.632.13-5.1-11.24-21.03-26.5-26.93-24.52-21.33-17.48-12.62-6.6606.6612.6217.4821.3324.5226.9326.521.0311.245.1-6.23-12.49-21.72-27.13-29.07-28.4-25.95-21.93-16.14-8.6108.6116.1421.9325.9528.429.0727.1321.7212.496.23-6.04-12.01-20.76-26.22-29.13-29.95-28.85-25.51-19.42-10.6010.619.4225.5128.8529.9529.1326.2220.7612.016.04-5.13-10.32-18.14-23.5-27.07-29.26-29.93-28.15-22.59-12.74012.7422.5928.1529.9329.2627.0723.518.1410.325.13-3.79-7.8-14.17-19.01-22.74-25.88-28.59-29.43-25.49-15.08015.0825.4929.4328.5925.8822.7419.0114.177.83.79-2.23-4.84-9.37-13.18-16.26-19.3-23.25-27.63-27.04-16.57016.5727.0427.6323.2519.316.2613.189.374.842.23-0.9-2.1-4.48-6.68-8.38-9.95-13.2-18.7-20.41-13.02013.0220.4118.713.29.958.386.684.482.10.9-0.376-0.852-1.96-3.06-3.85-4.56-5.67-15.47-23.98-12.91012.9123.9815.475.674.563.853.061.960.8520.376

Max. value at 3/4 of span - Nxy3·l4;h2=Nxy3·44;22=29.95 kN/m

Principal membrane forces, kN/m

Nmx;y=0.5·Nxx;y+Nyx;y,    ΔNx;y=0.5·Nxx;y-Nyx;y2+4·Nxyx;y2

Nmaxx;y=Nmx;y+ΔNx;y kN/m

Nminx;y=Nmx;y-ΔNx;y kN/m

## Solution with SAP 2000

Input data

Results

Normal membrane forces - Nx, kN/m (F11)

Normal membrane forces - Ny, kN/m (F22)

Shear membrane forces - Nxy, kN/m (F12)

Principal membrane forces - Nmax, kN/m (FMAX)

Principal membrane forces - Nmin, kN/m (FMIN)

## Analytical solution – Fourier series

Number of iterations - N=21

Original and Fourier functions for the load

Original and Fourier functions for the reaction in supports

Calculation of stresses

σxx;y=n=1NY2n;y·sinαn·x

Plot for σx at x = l/2

Bottom value - σxl2;0 m=σx4 m2;0 m=95.66 kN/ m

Top value - σxl2;h=σx4 m2;2 m=-147.56 kN/ m

σyx;y=-n=1Nαn2·Yn;y·sinαn·x

Middle top value - σyl2;h=σy4 m2;2 m=-92.32 kN/ m

τxyx;y=-n=1Nαn·Y1n;y·cosαn·x

Value at 3/4 of span - τxy3·l4;h2=τxy3·4 m4;2 m2=29.96 kN/ m

σmx;y=0.5·σxx;y+σyx;y,   Δσx;y=0.5·σxx;y-σyx;y2+4·τxyx;y2

σ1x;y=σmx;y+Δσx;y

σ2x;y=σmx;y-Δσx;y

## Comparison of the results

|  | Analytical | FEA Calcpad | FEA SAP 2000 |
| --- | --- | --- | --- |
| Nx,btm, kN/m | 95,66 | 92,26 | 92,46 |
| Nx,top, kN/m | -147,56 | -148,15 | -150 |
| Ny, kN/m | -92,32 | -104,3 | -103,97 |
| Nxy, kN/m | 29,96 | 29,95 | 30,91 |

Difference, %

|  | Analytical | FEA Calcpad | FEA SAP 2000 |
| --- | --- | --- | --- |
| Nx,btm, kN/m | 0,00% | -3,55% | -3,35% |
| Nx,top, kN/m | 0,00% | 0,40% | 1,65% |
| Ny, kN/m | 0,00% | 12,98% | 12,62% |
| Nxy, kN/m | 0,00% | -0,03% | 3,17% |